Thermal-Electric Field Distribution around Contact/Semi-insulating GaAs interfaces in Photo Conductive Semiconductor Switches

Xianyue Gu, Martin A. Gundersen

Department of Electrical Engineering – Electrophysics, University of Southern California,

Los Angeles, CA 90089-0271

Alan Mar

Sandia National Laboratories, Albuquerque, NM87185-1153

Charles W. Myles

# Department of Physics, Texas Tech University, Lubbock, TX 79409-1051

In the presence of an intensive current density, contact/semi-insulating (SI) GaAs interfacial defects can cause non-uniform field distribution. This can result in power dissipation around defects through the coupled nonlinear thermal-electric field, and can thus lead to a substantial temperature rise in the SI GaAs. Such heating likely plays role in switch damage. A 2D finite element GaAs switch simulation is presented which solves Poisson's equation and the thermal diffusion equation. It incorporates models of GaAs physical properties, including a carrier generation-recombination model and a field dependent carrier mobility model. Results of these simulations are compared to data on SI GaAs switches.

This work was supported by the Air Force Office of Scientific Research and the Army Research Office.

Author to whom correspondence should be directed.