Pulsed Power Conference, 2007

“Avalanche Injection and Lock-on in Photoconductive Semiconductor Switches”

H. P. Hjalmarson1, K. E. Kambour1, C. W. Myles2, R. P. Joshi3

1Sandia National Laboratories, Albuquerque, NM, USA
2Physics Department, Texas Tech University, Lubbock,TX, USA
3Department of Electrical and Computer Engineering, Old Dominion University, Norfolk, VA

Optically-triggered, high-power photoconductive semiconductor switches (PCSS's) using semi-insulating GaAs carry current in high carrier-density filaments.  This highly conductive mode of operation is called lock-on. The properties of these filaments can be explained by collective impact ionization theory in which energy redistribution by carrier-carrier scattering within the filament enhances the impact ionization. This allows these filaments to be sustained by fields which are relatively low compared to the bulk breakdown fields.  For GaAs, the sustaining lock-on field is approximately 4.5 kV/cm.  For this talk, a hydrodynamic implementation of the collective impact ionization theory is used to compute the temporal evolution of these filaments following optical triggering.  These continuum calculations are based on previous calculations in which the steady-state properties of filaments are computed using a Monte Carlo method to solve the Boltzmann equation.  The same method will be used to calculated avalanche injection effects in a GaAs avalanche photodiode.  The two modes of operation, lock-on and avalanche, will be compared and contrasted.  In addition, the effects of carrier recombination at defects will also be discussed.