UNIT 12 READING A

Capacitance of a Parallel Plate Capacitor

The capacitance of a capacitor is defined as

C=QΔV

For a parallel plate capacitor, the potential difference (the work needed to move a charge from one plate to another) between the plates is given by

ΔV=−WqΔV=−Fdq=−qEdqΔV=−Ed

where W is the work to move a charge from one plate to another, F = qE is the force of the electric field between the plates, and d is the distance between plates.

The potential difference is proportional to the distance, d, between the plates. Since the amount of charge on the plates, Q, depends on the area, A, of the plates, and it is reasonable to believe the capacitance on the area of the plates and the distance between the plates.

It can be shown mathematically and experimentally that capacitance is proportional to the ratio of the area of the plates to the distance between the plates, Ad. The capacitance of a parallel plate capacitor is

C=ε0Ad

where A is the area of the plates, d is the distance between the plates, and 0 is a constant. The value of 0 isε0=8.85×10−12C2Nm2.

Potential Energy of a Charged Capacitor

In the process of charging a capacitor, work is done to move the charges on the plates of the capacitor. The total work required to charge a capacitor can be calculated. It is found to be

W=Q22C

Since work is equal to a change in potential energy for conservative forces, the electrical potential energy of a capacitor increases by Q22C when it is charged. This potential energy would decrease and work would be done, if the capacitor were discharged.